CORE85

8085 Microprocessor Emulator & Simulator

Software Requirements Specification  (SRS)

| Document Version | 1.0 — Initial Release |
| --- | --- |
| Project Codename | Core85 |
| Target Platform | Windows 10+, macOS 12+, Ubuntu 22.04+ |
| Primary Language | C++17 / Qt 6.x |
| Document Status | Draft — Pending Technical Review |
| Target Audience | Students, Educators, Hobbyists, Computer Architecture Enthusiasts |
| Authors | Core85 Engineering Team |
| Last Updated | 22 April 2026 |

REVISION HISTORY

| Version | Date | Changes | Author |
| --- | --- | --- | --- |
| 1.0 | 22 Apr 2026 | Initial SRS document creation — full V1 scope defined. | Engineering Team |

# 1. Introduction

## 1.1  Purpose of This Document

This Software Requirements Specification (SRS) formally defines the complete functional, non-functional, and architectural requirements for Version 1.0 of the Core85 project. Core85 is a high-fidelity, cross-platform Intel 8085 microprocessor emulator and graphical simulator built in modern C++ and Qt.

This document serves as the single source of truth for the engineering team, QA, and stakeholders. All implementation decisions, test cases, and acceptance criteria must trace back to requirements listed herein.

## 1.2  Project Vision

The 8085 microprocessor remains a foundational component of computer architecture curricula worldwide, yet no open-source, cross-platform simulator exists that matches the accuracy and usability of commercial tools. Core85 aims to fill this gap by delivering:

- Cycle-accurate emulation of all 246 valid Intel 8085 instructions.

- A professional, interactive GUI that visualises every aspect of processor state in real-time.

- An integrated assembler that compiles mnemonics to machine code within the simulator.

- A fully packaged, installable desktop application for Windows, Linux, and macOS.

## 1.3  Scope of Version 1.0

Version 1.0 is strictly scoped to the desktop simulator. Features explicitly deferred to future versions include:

- WebAssembly (WASM) browser port  →  V1.5

- External peripheral chip support (8255 PPI, 8259 PIC)  →  V2.0

- Dedicated website (core85.org), gallery, and extended tutorials  →  V2.5

## 1.4  Intended Audience & Usage

- Students: Step through classic 8085 programs (sorting, arithmetic, string ops) instruction-by-instruction to build intuition.

- Educators: Use the simulator as a live teaching aid to demonstrate instruction execution, flag behaviour, and memory maps.

- Hobbyists: Write and test assembly programs in a risk-free software environment before deploying to real hardware.

- Researchers: Use the cycle-accurate timing to benchmark algorithms against the original 3 MHz hardware spec.

## 1.5  Definitions, Acronyms & Abbreviations

| Term / Acronym | Definition |
| --- | --- |
| 8085 | Intel 8085 8-bit microprocessor (1977). 40-pin DIP, 246 instructions, 3 MHz clock. |
| SRS | Software Requirements Specification — this document. |
| ALU | Arithmetic Logic Unit — the combinational circuit performing arithmetic and bitwise operations. |
| PC | Program Counter — 16-bit register holding the address of the next instruction. |
| SP | Stack Pointer — 16-bit register pointing to the top of the stack in memory. |
| T-State | One clock cycle of the 8085. Instructions take 4–18 T-states to execute. |
| ISR | Interrupt Service Routine — code executed upon receiving a hardware interrupt. |
| IME | Interrupt Master Enable — controlled by EI/DI instructions. |
| GUI | Graphical User Interface. |
| CMake | Cross-platform meta-build system used to generate native build files. |
| GTest | Google Test — C++ unit testing framework. |
| DXA | Device-independent eXtra-fine Units — 1440 DXA = 1 inch (used internally by DOCX). |

# 2. Technology Stack

The following technologies are mandated for Version 1.0. Substitution of any core component requires a formal change request approved by the lead architect.

| Component | Technology / Version | Rationale |
| --- | --- | --- |
| Core Emulator | C++17 (GCC 12+ / Clang 15+ / MSVC 2022) | Provides performance, deterministic memory layout, and access to low-level bit manipulation. C++17 features (std::variant, structured bindings, if-constexpr) reduce boilerplate in instruction dispatch. |
| GUI Framework | Qt 6.x (Qt Creator + Qt Designer) | Mature, cross-platform widget toolkit with native rendering on all target OSes. Qt's signal/slot mechanism cleanly decouples the emulator core from UI updates. |
| Build System | CMake 3.25+ | Industry-standard meta-build system. Generates Makefile, Ninja, and MSVC project files from a single source. Required for CPack packaging. |
| Unit Testing | Google Test (GTest) 1.14+ | Header-only integration with CMake via FetchContent. Allows mocking CPU state to validate individual instruction behaviour in isolation. |
| Packaging | CPack (bundled with CMake) | Generates .msi/.exe (NSIS/WiX on Windows), .dmg (macOS), and .deb/.AppImage (Linux) from a single CMakeLists.txt configuration block. |

# 3. Functional Requirements

## 3.1  Core Emulator Backend

The core emulator is a pure C++ state machine that must operate entirely independently of any UI code. It must be compilable as a standalone static library (libcore85.a / core85.lib). The GUI communicates with it exclusively via a well-defined C++ API (see Section 5.2).

### 3.1.1  Register File

| Req. ID | Title | Detailed Description | Priority | Phase |
| --- | --- | --- | --- | --- |
| REQ-CORE-01a | General-Purpose Registers | Implement all six 8-bit general-purpose registers: A (Accumulator), B, C, D, E, H, and L. Each must support independent read/write operations and pair access (BC, DE, HL) as 16-bit unsigned integers via helper methods getCombined() / setCombined(). | Critical | Ph.1 |
| REQ-CORE-01b | Special-Purpose Registers | Implement 16-bit registers: Program Counter (PC, reset to 0x0000 on RESET) and Stack Pointer (SP, initial value implementation-defined but conventionally set to 0xFFFF). Both must be accessible for direct read/write by the instruction execution engine. | Critical | Ph.1 |

### 3.1.2  Flag Register

| Req. ID | Title | Detailed Description | Priority | Phase |
| --- | --- | --- | --- | --- |
| REQ-CORE-02a | Flag Bits | Implement the 8-bit Flag register (F) with the following named bits at their exact positions: Sign (S, bit 7), Zero (Z, bit 6), Auxiliary Carry (AC, bit 4), Parity (P, bit 2), Carry (CY, bit 0). Bits 5, 3, and 1 are always 0. Bit 1 is always 1. | Critical | Ph.1 |
| REQ-CORE-02b | Sign Flag (S) | Set to 1 if the result of an arithmetic/logical operation has bit 7 = 1 (i.e., result is negative in two's complement). Cleared otherwise. | Critical | Ph.1 |
| REQ-CORE-02c | Zero Flag (Z) | Set to 1 if the result of an operation is exactly 0x00. Cleared otherwise. | Critical | Ph.1 |
| REQ-CORE-02d | Auxiliary Carry (AC) | Set to 1 if a carry occurs out of bit 3 into bit 4 during an addition, or a borrow occurs into bit 3 during subtraction. Required for DAA (Decimal Adjust Accumulator) instruction correctness. | Critical | Ph.1 |
| REQ-CORE-02e | Parity Flag (P) | Set to 1 if the number of 1-bits in the result byte is even (even parity). Cleared for odd parity. | Critical | Ph.1 |
| REQ-CORE-02f | Carry Flag (CY) | Set to 1 if a carry propagates out of bit 7 during addition, or a borrow is required during subtraction/comparison. Also used by rotate instructions (RAL, RAR, RLC, RRC). | Critical | Ph.1 |

### 3.1.3  Memory Subsystem

| Req. ID | Title | Detailed Description | Priority | Phase |
| --- | --- | --- | --- | --- |
| REQ-CORE-03a | 64KB Address Space | Allocate a contiguous 65,536-byte (64KB) array representing the unified memory map from address 0x0000 to 0xFFFF. All addresses are 16-bit unsigned integers. | Critical | Ph.1 |
| REQ-CORE-03b | Read/Write Interface | Provide readByte(uint16_t addr) and writeByte(uint16_t addr, uint8_t data) methods. Reading from uninitialized memory must return 0xFF (simulating unconnected bus lines). | Critical | Ph.1 |
| REQ-CORE-03c | ROM Protection (Configurable) | Allow optional designation of address ranges as read-only (ROM). Writes to ROM regions must be silently ignored (hardware-accurate behaviour). ROM ranges are configurable at load time. | High | Ph.1 |
| REQ-CORE-03d | Memory-Mapped I/O | Reserve a configurable address range for memory-mapped I/O (distinct from the port-based IN/OUT model). Default configuration uses only port-based I/O (see REQ-CORE-07). | Medium | Ph.2 |

### 3.1.4  Instruction Set

| Req. ID | Title | Detailed Description | Priority | Phase |
| --- | --- | --- | --- | --- |
| REQ-CORE-04a | Complete Instruction Coverage | Accurately decode and execute all 246 valid 8085 opcodes. The emulator must not silently skip or no-op any documented instruction. | Critical | Ph.2 |
| REQ-CORE-04b | Data Transfer Group | Implement all MOV, MVI, LXI, LDA, STA, LHLD, SHLD, LDAX, STAX, XCHG, XTHL, SPHL, PCHL, PUSH, POP instructions with cycle-accurate register and memory side-effects. | Critical | Ph.2 |
| REQ-CORE-04c | Arithmetic Group | Implement ADD, ADC, SUB, SBB, INR, DCR, INX, DCX, DAD, DAA, ADI, ACI, SUI, SBI instructions. All must update flags precisely per the Intel 8085 datasheet. | Critical | Ph.2 |
| REQ-CORE-04d | Logical Group | Implement ANA, ORA, XRA, CMA, CMC, STC, ANI, ORI, XRI, CPI instructions with correct flag behavior (note: ANA sets AC=1, CY=0). | Critical | Ph.2 |
| REQ-CORE-04e | Branch Group | Implement all unconditional (JMP, CALL, RET) and conditional jump/call/return instructions (JC, JNC, JZ, JNZ, JP, JM, JPE, JPO, and their CALL/RET equivalents). Implement RST n (n=0..7). | Critical | Ph.2 |
| REQ-CORE-04f | Machine Control Group | Implement NOP, HLT, EI, DI, SIM, RIM instructions. HLT must halt the fetch-decode-execute loop until an interrupt or RESET is received. | Critical | Ph.2 |
| REQ-CORE-04g | Undocumented Opcodes | Opcodes not defined by Intel (e.g., 0x08, 0x10) must be treated as NOP equivalents and a warning logged. They must not crash the emulator. | Low | Ph.2 |

### 3.1.5  Clock Cycle (T-State) Tracking

| Req. ID | Title | Detailed Description | Priority | Phase |
| --- | --- | --- | --- | --- |
| REQ-CORE-05a | Per-Instruction T-State Count | Each instruction execution must increment a 64-bit cycle counter by the exact number of T-states defined in the Intel 8085 datasheet. Branch-taken vs. branch-not-taken paths must use their respective T-state counts. | Critical | Ph.2 |
| REQ-CORE-05b | Cycle Counter API | Expose getElapsedCycles() : uint64_t and resetCycles() methods on the CPU class. The GUI uses this for timing display and performance profiling. | High | Ph.2 |

### 3.1.6  Interrupt Subsystem

| Req. ID | Title | Detailed Description | Priority | Phase |
| --- | --- | --- | --- | --- |
| REQ-CORE-06a | TRAP (RST 4.5) | Non-maskable interrupt. Cannot be disabled by DI. Triggers unconditionally when asserted. Vectors to address 0x0024. Simulated via raiseInterrupt(TRAP) API call. | Critical | Ph.2 |
| REQ-CORE-06b | RST 7.5 | Maskable, edge-triggered interrupt. Governed by the M7.5 bit in SIM and IME. Vectors to 0x003C. | Critical | Ph.2 |
| REQ-CORE-06c | RST 6.5 | Maskable, level-triggered interrupt. Governed by the M6.5 bit in SIM and IME. Vectors to 0x0034. | Critical | Ph.2 |
| REQ-CORE-06d | RST 5.5 | Maskable, level-triggered interrupt. Governed by the M5.5 bit in SIM and IME. Vectors to 0x002C. | Critical | Ph.2 |
| REQ-CORE-06e | INTR | General maskable interrupt. When acknowledged, the CPU issues an INTA signal; the external device must place an RST instruction on the data bus. Simulated by specifying the RST opcode in the raiseInterrupt(INTR, uint8_t rstOpcode) call. | High | Ph.2 |
| REQ-CORE-06f | Interrupt Latency | Interrupts are checked at the end of every instruction cycle. The minimum latency is therefore the remaining T-states of the currently-executing instruction. | High | Ph.2 |

### 3.1.7  I/O Port Interface

| Req. ID | Title | Detailed Description | Priority | Phase |
| --- | --- | --- | --- | --- |
| REQ-CORE-07a | 256 Input Ports | Maintain a 256-element array of uint8_t representing input port values (port 0x00 to 0xFF). The IN r instruction reads from the port addressed by register C (or immediate byte in INI variant) and loads the value into the accumulator. | Critical | Ph.1 |
| REQ-CORE-07b | 256 Output Ports | Maintain a 256-element array of uint8_t representing output port values. The OUT r instruction writes the accumulator value to the addressed port. The GUI polls this array to update peripheral widgets. | Critical | Ph.1 |
| REQ-CORE-07c | Port Change Callbacks | Provide an optional callback mechanism (std::function<void(uint8_t port, uint8_t value)>) triggered whenever an output port is written. This enables the GUI to respond to OUT instructions without polling. | High | Ph.4 |

## 3.2  Graphical Simulator Frontend

The frontend is a Qt 6 application that wraps the core library. It must load dynamically or link statically against libcore85 and communicate exclusively via the Core85 Public API (Section 5.2). No Qt headers may be included in src/core/.

### 3.2.1  Main Layout

| Req. ID | Title | Detailed Description | Priority | Phase |
| --- | --- | --- | --- | --- |
| REQ-GUI-01a | Unified Dashboard | The main window must present a single, docked layout containing: Code Editor panel (left), Execution Control toolbar (top), Register & Flag Viewer (right-top), Memory Viewer (right-bottom), and I/O Peripheral Panel (bottom). Panels must be resizable via drag handles. | Critical | Ph.4 |
| REQ-GUI-01b | Dark & Light Themes | The application must ship with both a Light and Dark theme, selectable from Preferences. Theme preference must persist across sessions via QSettings. | High | Ph.4 |
| REQ-GUI-01c | Status Bar | A persistent status bar at the bottom of the window must display: current PC value, T-state count, execution state (Running / Paused / Halted), and assembler/emulator error messages. | High | Ph.4 |

### 3.2.2  Code Editor & Integrated Assembler

| Req. ID | Title | Detailed Description | Priority | Phase |
| --- | --- | --- | --- | --- |
| REQ-GUI-02a | Text Editor Widget | Embed a line-numbered text editor (QPlainTextEdit subclass) supporting multi-line assembly source code. The editor must display line numbers in the gutter. | Critical | Ph.3 |
| REQ-GUI-02b | Syntax Highlighting | Apply real-time syntax highlighting via a QSyntaxHighlighter subclass: opcodes (blue, bold), register names (teal), immediate values (orange), labels (green), comments (; prefix, italic gray), and directives (.ORG, .EQU, DB, DW in purple). | High | Ph.3 |
| REQ-GUI-02c | Assembler Engine | Implement a two-pass assembler in src/core/assembler.cpp. Pass 1 collects labels and .EQU constants into a symbol table. Pass 2 generates machine code bytes. Supported directives: .ORG (set load address), DB (define byte), DW (define word), .EQU (symbolic constant). | Critical | Ph.3 |
| REQ-GUI-02d | Assembler Error Reporting | On assembly failure, highlight the offending line in red and display the error message (line number, error type, token) in the status bar and a dockable error console. | High | Ph.3 |
| REQ-GUI-02e | Load & Assemble Button | A prominently placed Assemble & Load button must trigger assembly and, on success, write the resulting machine code to memory starting at the .ORG address, reset the PC, and clear all registers. | Critical | Ph.4 |
| REQ-GUI-02f | Execution Cursor | During Step or Run modes, the currently-executing source line must be highlighted with a yellow background in the editor. This requires maintaining a PC-to-line-number mapping table built during assembly. | High | Ph.4 |
| REQ-GUI-02g | File I/O | Support Open, Save, and Save As operations for .asm source files. Support loading pre-assembled .hex (Intel HEX format) files directly into memory. | High | Ph.4 |

### 3.2.3  Execution Controls

| Req. ID | Title | Detailed Description | Priority | Phase |
| --- | --- | --- | --- | --- |
| REQ-GUI-03a | Run | Execute instructions in a separate QThread at full emulated speed. The GUI must receive state-update signals via Qt's signal/slot mechanism at a configurable refresh rate (default: 60 Hz). The emulator thread must never block the UI thread. | Critical | Ph.4 |
| REQ-GUI-03b | Pause | Interrupt the execution thread between instructions. The CPU state at the point of pause must be immediately reflected in the GUI. Pause must be responsive within 1 instruction cycle (< 1 ms at 3 MHz equivalent). | Critical | Ph.4 |
| REQ-GUI-03c | Step Into | Execute exactly one instruction from the current PC, then pause. The GUI must update all register, flag, and memory views synchronously before returning control to the user. | Critical | Ph.4 |
| REQ-GUI-03d | Step Over | Execute one instruction; if that instruction is a CALL, execute until the matching RET is reached, then pause. Implemented by setting a temporary breakpoint at PC + instruction_length before running. | High | Ph.4 |
| REQ-GUI-03e | Reset | Set PC to 0x0000, clear all registers to 0x00, clear all flags to 0, reset the cycle counter, and restore memory to the last-assembled state. | Critical | Ph.4 |
| REQ-GUI-03f | Breakpoints | Allow users to set/clear breakpoints by clicking in the editor gutter. Run mode must halt execution when PC reaches a breakpointed address. | High | Ph.4 |
| REQ-GUI-03g | Execution Speed Control | Provide a slider or dropdown to adjust emulation speed from 1 instruction/step (manual) to unconstrained (maximum host CPU speed). | Medium | Ph.4 |

### 3.2.4  Register & Flag Viewer

| Req. ID | Title | Detailed Description | Priority | Phase |
| --- | --- | --- | --- | --- |
| REQ-GUI-04a | Register Display | Display all 8 registers (A, B, C, D, E, H, L, F) and the two 16-bit registers (SP, PC) with their current values in both hexadecimal and unsigned decimal. Register pairs (BC, DE, HL) must also be shown as combined 16-bit values. | Critical | Ph.4 |
| REQ-GUI-04b | Change Highlighting | Registers whose values changed during the last instruction execution must be highlighted (e.g., yellow background) and the highlight must fade or persist until the next step. This provides immediate visual feedback. | High | Ph.4 |
| REQ-GUI-04c | Flag Display | Display the five flags (S, Z, AC, P, CY) as individual labelled bit indicators showing 0 or 1. Set flags must use a visually distinct colour (e.g., green for SET, grey for CLEAR). | Critical | Ph.4 |
| REQ-GUI-04d | Inline Editing | Allow users to directly edit any register value by double-clicking the value field. Input must be accepted in both hexadecimal (0x prefix or H suffix) and decimal. Invalid input must be rejected with an error tooltip. | High | Ph.4 |

### 3.2.5  Memory Viewer

| Req. ID | Title | Detailed Description | Priority | Phase |
| --- | --- | --- | --- | --- |
| REQ-GUI-05a | Hex Grid Display | Display the 64KB memory space as a scrollable grid: 16 bytes per row, address column on the left (format: 0x0000), hex values in the centre, and ASCII representation on the right (printable characters, dot for non-printable). | Critical | Ph.4 |
| REQ-GUI-05b | Inline Editing | Allow users to click any hex cell to edit it directly. Changes must be written to the emulator memory immediately. The ASCII column must update in sync. | High | Ph.4 |
| REQ-GUI-05c | Go-To Address | Provide a Go To Address input field that scrolls the memory view to a specified 16-bit address. | High | Ph.4 |
| REQ-GUI-05d | PC Tracking | Optionally auto-scroll the memory view to keep the current PC address visible during stepping. Configurable via a toolbar toggle. | Medium | Ph.4 |
| REQ-GUI-05e | Modified Cell Highlighting | Cells written to during the last instruction must be highlighted in a distinct colour (e.g., light red background) to visualise memory side-effects. | High | Ph.4 |

### 3.2.6  I/O Peripheral Simulation

| Req. ID | Title | Detailed Description | Priority | Phase |
| --- | --- | --- | --- | --- |
| REQ-GUI-06a | Toggle Switch Input Panel | Provide a panel of 8 toggle switches wired to a configurable input port (default: port 0x00). Each switch toggles its corresponding bit in the port value. An IN 0x00 instruction must read the current switch state. | Critical | Ph.4 |
| REQ-GUI-06b | LED Output Panel | Provide a panel of 8 LED indicators wired to a configurable output port (default: port 0x01). The LEDs must reflect the value written by an OUT 0x01 instruction in real-time via the port-change callback (REQ-CORE-07c). | Critical | Ph.4 |
| REQ-GUI-06c | 7-Segment Display | Provide a single 7-segment display widget wired to a configurable output port (default: port 0x02). The widget must decode and render the standard 7-segment encoding written to the port. | High | Ph.4 |
| REQ-GUI-06d | Port Address Configuration | Allow users to remap the I/O widgets to any port address via a configuration panel. The mapping must persist in the project file. | Medium | Ph.4 |

# 4. Non-Functional Requirements

| NFR ID | Category | Requirement | Acceptance Criteria |
| --- | --- | --- | --- |
| NFR-01 | Performance | The core emulator must execute instructions at a sustained rate of at least 3,000,000 T-states/second (matching the original 3 MHz clock) on a modern mid-range PC. Peak throughput should reach 100M+ T-states/second. | Measured via bench_core unit test on reference hardware (Intel Core i5, 8GB RAM). |
| NFR-02 | Portability | The codebase must compile with zero errors and zero warnings on GCC 12+, Clang 15+, and MSVC 2022 in C++17 mode using -Wall -Wextra on GCC/Clang and /W4 on MSVC. | CI pipeline must pass on all three compilers. |
| NFR-03 | Separation of Concerns | src/core/ must compile as a standalone static library without any Qt dependency. CMake target core85_lib must have no Qt linkage. | cmake --build with Qt removed must succeed for core target. |
| NFR-04 | Test Coverage | All 246 instructions must have at least one dedicated unit test asserting correct register state, flag state, and T-state count after execution. | GTest suite must report >= 246 passing instruction tests. |
| NFR-05 | GUI Responsiveness | The UI thread must never block for more than 16 ms (60 FPS target) regardless of emulator speed. Emulation runs on a dedicated QThread. | No frame drops below 30 FPS during Run mode, measured via Qt performance profiler. |
| NFR-06 | Memory Safety | No use of raw new/delete in new code. Use smart pointers (std::unique_ptr, std::shared_ptr) and STL containers. Valgrind / AddressSanitizer must report zero memory leaks on the test suite. | CI run with -fsanitize=address must be clean. |
| NFR-07 | Usability | A user familiar with 8085 assembly must be able to write, assemble, and step through a 10-instruction program within 5 minutes of first launch, without consulting documentation. | Validated via a 5-person usability test session. |
| NFR-08 | Package Size | The installer must not exceed 150 MB on any platform. The runtime dependency on Qt must be satisfied by bundling only required Qt modules. | Installer sizes verified on all three platforms post-packaging. |

# 5. Architecture & Project Structure

## 5.1  Directory Layout

The following canonical project structure must be maintained throughout development. Deviations require a pull request description explaining the structural change.

| Path | Purpose |
| --- | --- |
| Core85/CMakeLists.txt | Master build script. Defines targets: core85_lib (static), core85_gui (executable), core85_tests. |
| src/core/cpu.cpp¦h | CPU class: register file, ALU, Fetch/Decode/Execute loop, interrupt controller. Exposes Core85 Public API. |
| src/core/memory.cpp¦h | Memory class: 64KB array, read/write with ROM guard, memory-mapped I/O hooks. |
| src/core/assembler.cpp¦h | Two-pass assembler: tokenizer, symbol table, code generator, Intel HEX output. |
| src/core/io_bus.cpp¦h | I/O Bus: 256-port input/output arrays, callback registry for OUT notifications. |
| src/gui/main.cpp | Qt application entry point: QApplication setup, MainWindow instantiation. |
| src/gui/mainwindow.ui | Qt Designer .ui file describing the docked main layout. |
| src/gui/mainwindow.cpp¦h | MainWindow class: wires GUI signals to core API calls, owns the emulator QThread. |
| src/gui/components/ | Subdirectory for reusable Qt widgets: MemoryViewer, RegisterViewer, IOPanel, CodeEditor. |
| tests/test_arithmetic.cpp | GTest suite: ADD, ADC, SUB, SBB, INR, DCR, DAA — verifies flags and register state. |
| tests/test_logical.cpp | GTest suite: ANA, ORA, XRA, CMA, CMC, STC, rotation instructions. |
| tests/test_branch.cpp | GTest suite: JMP, CALL, RET, all conditional variants, RST vectors. |
| tests/test_memory.cpp | GTest suite: byte read/write, ROM protection, boundary addresses (0x0000, 0xFFFF). |
| tests/test_interrupts.cpp | GTest suite: TRAP, RST 7.5/6.5/5.5, INTR, EI/DI interaction. |
| docs/ | API documentation (Doxygen-generated) and user manual source. |

## 5.2  Core85 Public API (C++ Interface)

The following methods constitute the formal contract between the GUI and the core library. All methods are members of the Core85::CPU class defined in src/core/cpu.h.

| Method Signature | Description |
| --- | --- |
| void step() | Execute one instruction from PC. Update registers, flags, memory, and cycle counter. Returns after instruction completes. |
| void reset() | Reset PC to 0x0000, clear all registers and flags, reset cycle counter. Does not modify memory. |
| CPUState getState() const | Return a snapshot struct containing all register values, flag register, PC, SP, and elapsed cycles. |
| void loadProgram(span<uint8_t>, uint16_t addr) | Write byte array into memory starting at addr. Sets PC to addr. |
| void setInputPort(uint8_t port, uint8_t val) | Set the value on input port [port] to val. Read by IN instructions. |
| uint8_t getOutputPort(uint8_t port) const | Read the value last written to output port [port] by an OUT instruction. |
| void raiseInterrupt(InterruptType, uint8_t rst=0) | Assert a hardware interrupt. For INTR, rst specifies the RST opcode. |
| bool isHalted() const | Returns true if the CPU is in HLT state. |
| void setOnOutputWrite(callback) | Register a callback invoked when any OUT instruction writes to a port. |

# 6. Development Milestones — Version 1.0

| Phase | Name | Duration | Deliverables & Exit Criteria | Requirements Covered |
| --- | --- | --- | --- | --- |
| Ph. 1 | The Brain | Weeks 1–3 | CPU class with register file + Memory class. GTest suite with >= 20 passing tests covering register read/write, memory boundary conditions, and flag initialisation. | REQ-CORE-01, 02, 03, 07; NFR-01, 03, 04, 06 |
| Ph. 2 | The Logic | Weeks 3–6 | All 246 instructions implemented and tested. Each instruction has a corresponding GTest case validating register state, flags, and T-state count. Interrupt controller complete. | REQ-CORE-04, 05, 06; NFR-01, 04 |
| Ph. 3 | The Translator | Weeks 7–8 | Two-pass assembler supporting all mnemonics, .ORG, DB, DW, .EQU, and labels. Assembler must correctly translate the provided reference programs in /tests/asm/. | REQ-GUI-02c, 02d, 02g |
| Ph. 4 | The Face | Weeks 9–12 | Fully functional Qt GUI wired to core library. All REQ-GUI-* requirements implemented. User can write, assemble, step, and inspect state. Both Light and Dark themes functional. | All REQ-GUI-*; NFR-05, 07 |
| Ph. 5 | The Package | Week 13 | CMake/CPack configuration producing installable artefacts on all three platforms. Installers smoke-tested. Version 1.0 tag created. Release notes published. | NFR-02, 08 |

# 7. Future Scope (Post V1.0)

The following features are explicitly out of scope for Version 1.0 and are documented here to avoid scope creep. They will be re-evaluated during the Version 1.5 / 2.0 planning cycle.

| Version | Feature | Description | Dependency |
| --- | --- | --- | --- |
| V1.5 | WASM Browser Port | Compile the core emulator to WebAssembly via Emscripten. Provide a React/TypeScript frontend at core85.org. The core's strict separation of concerns (NFR-03) is the key enabler. | NFR-03 fulfilled; Emscripten toolchain |
| V2.0 | 8255 PPI Support | Attach a software model of the Intel 8255 Programmable Peripheral Interface to I/O ports 0xF0–0xF3. Enables interfacing with keyboard matrix and parallel port simulations. | V1.0 I/O port framework |
| V2.0 | 8259 PIC Support | Attach a software model of the Intel 8259 Programmable Interrupt Controller. Required for cascaded interrupt priority management demos. | V1.0 Interrupt subsystem |
| V2.5 | core85.org Website | Dedicated project website with interactive browser emulator, tutorial series, documentation portal, and a gallery of community-submitted assembly programs. | V1.5 WASM port |
| V2.5 | Disassembler View | A panel displaying disassembled mnemonics for the current memory contents, enabling reverse-engineering workflows and analysis of pre-assembled HEX files. | V1.0 assembler symbol table |

# 8. Risks & Mitigations

| Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| Flag behaviour edge cases (AC, DAA) are subtly incorrect | Medium | High | Cross-validate against a known-good 8085 emulator (e.g., Sim8085) and against the Intel 8085 Programmer's Manual errata. |
| Qt licensing compliance for commercial distribution | Low | High | Use Qt LGPL edition; dynamically link Qt libraries to comply with LGPL terms. Document compliance in NOTICES.md. |
| CMake/CPack packaging fails on one platform | Medium | Medium | Add all three platforms to CI (GitHub Actions matrix). Run CPack smoke tests on every merge to main. |
| Assembler correctness for complex addressing modes | Medium | High | Maintain a corpus of reference programs with known-correct hex output. Regression-test assembler output on every commit. |
| GUI performance degrades for large memory edits | Low | Medium | Use a virtual QAbstractItemModel for the memory viewer so only visible rows are rendered. |

End of Core85 Software Requirements Specification v1.0